Inclusions List

▪ Supply and install a single fire pumps with supply duties of 22L|S @ 1000KPA

▪ Supply and install a round galvanised fire water storage tank with an effective capacity of KL

▪ Supply and install both hydrant and sprinkler booster assembly including tank suction and attack hydrants.

▪ Supply and install sprinkler control valves as shown on the schematic drawing.

▪ Supply and install hydrant pipework and risers, hydrant landing valves and hose reels.

▪ Supply and install a hydrant test drain including a storz connection point, hydrant test drain to terminate as per schematic drawing.

▪ Supply and install 50mm test drain for the sprinkler control valves.

▪ Supply and install residential semi recessed sprinkler heads, concealed space sprinklers, sidewall sprinklers and exposed sprinklers as shown on drawings provided.

▪ Supply and install sprinkler heads to the bin chutes and a head protecting the bottom off the lift shafts which has a separate monitored control valve to isolate it.

▪ Provide a single hydrostatic pressure test to each level after pipe work is roughed in.

▪ Provide fire rating off all required penetrations and certification.

▪ Provide full system design, workshop drawings for approval in CAD and PDF formats and as-builds once install is complete.

▪ Provide sprinkler and hydrant block plans and emergency instructions at the control valves.

▪ Provide operations & maintenance manuals.

▪ Conduct final commissioning and testing on completion and provide compliance certification when required.

▪ Provide12 months maintenance.

▪ 12 Months warranty.

Exclusions List

▪ We have made no allowances for any incoming towns mains works, water meters or back flow prevention devices.

▪ We have made no allowances to supply the water feed for the tank, fire pumps or booster assemblies.

▪ We have made no allowance to carry out any underground works associated with the fire pumps, tanks, hydrants, boosters, hose reels or sprinkler system.

▪ We have made no allowances to provide a drainage point for the fire pumps discharge or the testing of the hydrants or sprinkler system.

▪ We have made no allowance to supply a concrete tank, sealing or lining off the tank, penetrations, access ladders, tank overflows, tank level indicators or any pipework within the tank.

▪ We have made no allowance to provide a concrete slab for either the fire water storage tank, fire pumproom or booster assemblies.

▪ We have made no allowance to provide a fire pump room or booster assembly enclosure unless specified in our quote.

▪ We have made no allowance to terminate the diesel exhausts other than within the carpark or an external location close to the fire pump room.

▪ We have made no allowance for any extra costs involved in the project until a copy of the latest flows and pressures are provided or the project is a D&C.

▪ We have made no allowance to provide a design in 3D.

▪ We have made no allowance to discharge any fire water under the new AS3500 amendments.

▪ We have made no allowance for special hazard protection for the car charging ports within the carpark if required.

▪ We have made no allowance to paint exposed pipework other than a signal red finish or provide the pipe in galvanized steel.

▪ We have made no allowance for any changes to the works program that requires any out of sequence works.

▪ We have made no allowance to provide a spotter for any works carried out on an elevated works platform.

▪ We have made no allowance for any council, brigade, government or inspection fees.

▪ We have made no allowance to work more than 8 hours a day Monday to Friday.